##### OPTICAL HEART RATE DETECTION

##### A SYNOPSIS

###### Submitted by

##### Abhishek Patwa

##### Garvita Jain

##### Neelam Kukreja

##### Bachelor of Technology

IN

Computer Science and Information Technology

TO

School of Computer Science and Information Technology

SYMBIOSIS UNIVERSITY OF APPLIED SCIENCES, INDORE

Under the guidance of

##### NAME OF THE FACULTY MENTOR

MAY 2021

# Guidelines for Synopsis

# Title: Optical Heart Rate Detection via Webcam

Problem Definition: Heart rate in the current age is monitored using pulse oximetry sensors that may be worn on the fingertip or earlobe. These sensors are not convenient for long-term wear and the pressure can become uncomfortable over time. In addition to the discomforts of traditional pulse measurement devices, these devices can damage the fragile skin of premature newborns or elderly people. For these populations especially, a non-contact means of detecting pulse could be very beneficial. Non-contact heart rate measurement through a simple webcam or phone camera would also aid telemedicine and allow the average person to track their heart rate without purchasing special equipment

# Objectives: To develop a virtual heart rate measurement system which can be easily accessed through any system’s webcam.

Scope: As part of the recent gain in popularity of fitness apps and the quantified self, regular non-obtrusive monitoring through a computer or phone camera may help detect changes in a person’s heart rate over time and indicate changing health or fitness. Heart rate can be detected without contact through photo-plethysmography (PPG), which measures variations in blood volume by detecting changes in light reflectance or transmission throughout the cardiovascular pulse cycle. PPG is usually performed with dedicated light sources with red or infrared wavelengths, as is the case for pulse oximetry sensors. Plethysmography signal could also be detected in video from a regular color camera.  The ALGORITHM

- Perform face detection on one frame of the video

- Decompose this frame into sub frames using the Gaussian pyramid decomposition algorithm. Thus the highest level of the pyramid has low pass information.

- Filter each level of the pyramid using a Band Pass Filter, with cut-off frequency given as [(hifreq*frame rate/sampling rate)±1] .

- Magnify the intensities of the frequency band of interest by a magnification factor.

- Collapse the pyramid into a single frame.

- Repeat the above 5 steps for all the remaining frames of the video.

# Proposed Methodology/ Planning of work

# Detecting heart rate in video consists of three main steps, which are detailed in the following sections. First, the facial region must be detected in each frame of the video since the face is the only portion of the frame that will contain heart rate information. Second, the desired region of interest (ROI) within the face bounding box must be chosen. And third, the plethysmography signal must be extracted from the change in pixel colors within the ROI over time and analyzed to determine the prominent frequency within the heart rate range).

# Facilities required for proposed work:

The software is implemented on a Windows environment using Python progamming language aided by three libraries:  OpenCV, Numpy and Scipy. The OpenCV library provides the necessary resources to the facial detection algorithm and the frame capture through the webcam connected to a computer. Numpy and Scipy are used for optimization, stats and signal processing. The software developed does not rely on any OS specific functionality, and thus can be ported to a Linux environment

# Timeline: The development of the software shall be completed in the duration of 2 months.

# Team: The role of documentation regarding the project as well as running test cases shall be taken on by Abhishek Patwa. The role of development of the core software shall be taken on by Neelam Kukreja. The role of compiling datasets and assisting in the development of the software shall be taken on by Garvita Jain.

# Bibliography

# https://ep.liu.se/ecp/129/002/ecp16129002.pdf , H. Rahman, M.U. Ahmed, S. Begum, P. Funk

# S. Cook, M. Togni, M. C. Schaub, P. Wenaweser, and O. M. Hess, “High heart rate: a cardiovascular risk factor?” Eur. Heart J. 27(20), 2387–2393 (2006).

# I. Pavlidis, J. Dowdall, N. Sun, C. Puri, J. Fei, and M. Garbey, “Interacting with human physiology,” Comput. Vis. Image Underst. 108(1-2), 150–170 (2007).

# M. Garbey, N. Sun, A. Merla, and I. Pavlidis, “Contact-free measurement of cardiac pulse based on the analysis of thermal imagery,” IEEE Trans. Biomed. Eng. 54(8), 1418–1426 (2007).

# J. Fei, and I. Pavlidis, “Thermistor at a Distance: Unobtrusive Measurement of Breathing,” IEEE Trans. Biomed. Eng. 57(4), 988–998 (2010).

# F. P. Wieringa, F. Mastik, and A. F. van der Steen, “Contactless multiple wavelength photoplethysmographic imaging: a first step toward “SpO2 camera” technology,” Ann. Biomed. Eng. 33(8), 1034–1041 (2005).

# K. Humphreys, T. Ward, and C. Markham, “Noncontact simultaneous dual wavelength photoplethysmography: a further step toward noncontact pulse oximetry,” Rev. Sci. Instrum. 78(4), 044304 (2007).

# C. Takano, and Y. Ohta, “Heart rate measurement based on a time-lapse image,” Med. Eng. Phys. 29(8), 853– 857 (2007).

# S. Hu, J. Zheng, V. Chouliaras, and R. Summers, “Feasibility of imaging photoplethysmography,” in Proceedings of IEEE Conference on BioMedical Engineering and Informatics (IEEE, 2008), pp. 72–75.

# W. Verkruysse, L. O. Svaasand, and J. S. Nelson, “Remote plethysmographic imaging using ambient light,” Opt. Express 16(26), 21434–21445 (2008).

# J. Allen, “Photoplethysmography and its application in clinical physiological measurement,” Physiol. Meas. 28(3), R1–R39 (2007).

# S. Rhee, B. H. Yang, and H. H. Asada, “Artifact-resistant power-efficient design of finger-ring plethysmographic sensors,” IEEE Trans. Biomed. Eng. 48(7), 795–805 (2001).

# M. Z. Poh, N. C. Swenson, and R. Picard, “Motion-tolerant magnetic earring sensor and wireless earpiece for wearable photoplethysmography,” IEEE Trans Inf Technol Biomed (Epub 2010 Feb).

Table of Contents

S.N.	Content 									      Page No.

1.	Title	2

2.	Problem Definition	2

3.	Objective	2

4.	Scope	2

5.	Literature Review / Feasibility Study	3

6.	Proposed Methodology 	5

7.	Expected Outcomes 	5

8.	Facilities required 	5

9.	Timeline	6

10.	Team	7

Bibliography	7